EXPERIMENT – 5

DESIGN AND SIMULATION OF UJT RELAXATION OSCILLATOR USING LTSPICE

OBJECTIVES

- Analysis of UJT Relaxation Oscillator.

- Determine Bandwidth

- Determine Duty Cycle

## EXPERIMENT – 5.1

FREQUENCY ANALYSIS OF UJT RELAXATION OSCILLATOR USING LTSPICE

### After completion of this experiment, students will be able to:

- Understand the construction and operating principle of a Unijunction Transistor (UJT).

- Explain the working of a UJT Relaxation Oscillator.

- Verify the generation of oscillations using a UJT-based oscillator circuit.

- Observe the charging and discharging characteristics of the timing capacitor.

- Measure the oscillation frequency and compare it with the theoretical value.

- Analyze the effect of resistance and capacitance values on the oscillation frequency.

- Identify and interpret the sawtooth waveform and trigger pulse waveform generated by the circuit.

- Determine the role of the intrinsic stand-off ratio (η)(\eta)(η) in oscillator operation.

- Plot and analyze the output waveforms using a CRO/oscilloscope or simulation tool.

- Apply UJT oscillators in timing, triggering, pulse generation, and sweep generation applications.

### Aim

To verify the operation of a UJT Relaxation Oscillator.

## THEORY

The UJT relaxation oscillator generates non-sinusoidal oscillations. Operation:

  - Capacitor charges exponentially.

  - UJT triggers at peak voltage.

  - Capacitor discharges rapidly.

  - Cycle repeats continuously.

## DESIGN FORMULA:

## PROCEDURE

- Draw UJT oscillator in LTSpice.

- Configure RC network.

- Run transient analysis.

- Observe sawtooth waveform.

- Measure oscillation frequency.

Circuit diagram:

### Procedure:

  - Connections are made as shown in the circuit diagram.

  - The DC power supply is switched ON.

  - The output waveform is displayed on the CRO.

  - The peak-to-peak Amplitude and time period of the sine wave is noted.

  - The graph of output waveform is drawn.

### Model Graph:

## OBSERVATION TABLE

| Waveform | Observation |
| --- | --- |
| Amplitude | 11.4V |
| Time period | 38.06ms |
| Frequency | 26.27 Hz |

### Result:

UJT Relaxation Oscillator using LTSpice was completed and output obtained.

## EXPERIMENT – 5.2

BANDWIDTH ANALYSIS OF UJT RELAXATION OSCILLATOR USING LTSPICE

### After completion of this experiment, students will be able to:

- Understand the operating principle of a UJT Relaxation Oscillator and its frequency-determining mechanism.

- Simulate a UJT Relaxation Oscillator using LTSpice software.

- Analyze the charging and discharging behavior of the timing capacitor in the oscillator circuit.

- Measure the oscillation frequency from the simulated output waveform.

- Compare theoretical and simulated frequency values of the oscillator.

- Investigate the effect of varying resistance (R)(R)(R) and capacitance (C)(C)(C) values on the oscillation frequency.

- Observe and interpret the generated sawtooth and pulse waveforms using LTSpice.

- Plot and analyze the relationship between frequency and RC time constant.

- Apply LTSpice simulation tools for frequency analysis and performance evaluation of relaxation oscillators.

### Aim

To verify frequency variation with RC values.

Circuit diagram:

### Procedure:

  - Connections are made as shown in the circuit diagram.

  - The DC power supply is switched ON.

  - The output waveform is displayed on the CRO.

  - The peak-to-peak Amplitude and time period of the sine wave is noted.

  - The graph of output waveform is drawn.

### Model Graph:

## OBSERVATION TABLE

| Waveform | Observation |
| --- | --- |
| Amplitude | 11.4V |
| Time period | 38.06ms |
| Frequency | 26.27 Hz |
| Bandwidth | 11.16 Hz |

### Result:

Thus the bandwidth of UJT Relaxation Oscillator using LTSpice was completed and output obtained.

## EXPERIMENT – 5.3

WAVEFORM ANALYSIS OF UJT RELAXATION OSCILLATOR WITH DUTY CYCLE USING LTSPICE

### After completion of this experiment, students will be able to:

- Understand the operating principle of a UJT Relaxation Oscillator and its frequency-determining mechanism.

- Simulate a UJT Relaxation Oscillator using LTSpice software.

- Analyze the charging and discharging behavior of the timing capacitor in the oscillator circuit.

- Measure the oscillation frequency from the simulated output waveform.

- Compare theoretical and simulated frequency values of the oscillator.

- Investigate the effect of varying resistance (R)(R)(R) and capacitance (C)(C)(C) values on the oscillation frequency.

- Observe and interpret the generated sawtooth and pulse waveforms using LTSpice.

- Plot and analyze the relationship between frequency and RC time constant.

- Apply LTSpice simulation tools for frequency analysis and performance evaluation of relaxation oscillators.

# Duty Cycle

  - Ratio of pulse width to total period:

### Circuit diagram:

### Procedure:

- Connections are made as shown in the circuit diagram.

- The DC power supply is switched ON.

- The output waveform is displayed on the CRO.

- The peak-to-peak Amplitude and time period of the sine wave is noted.

- The graph of output waveform is drawn.

### Model Graph:

## OBSERVATION TABLE

| Waveform | THEORITICAL | PRACTICAL |
| --- | --- | --- |
| Pulse width | 28 | 27.69 |
| period | 39 | 38.06 |
| Duty Cycle | 73 | 72.75 |

### Result:

UJT Relaxation Oscillator using LTSpice was completed and output obtained.